<Outline>

<1: Optical architecture search (OAS): problem formulation>

<2: Architecture>

<2.1: Physical optical system>

<2.2: Optical element>

<2.3: Object volume>

<2.4: Architecture>

<2.5: Search space>

<3: Task & dataset>

<3.1: Task>

<3.2: Dataset>

<4: Loss function>

<5: Example tasks>

Section 1: Optical architecture search (OAS): problem formulation

Fig.1. Overview of the proposed optical architecture search.

Let A be the space of all possible (or considered) optical architectures (See Fig1). Let a∈A be an optical architecture (see Section X). Let T be the imaging task of interest (see section X). Then we define the problem of optical architecture search as finding an approximate solution to the following optimization.

aT*=mina∈ALa,w*a,DTval  s.t. w*a=minw(a)La,w(a),DTtrain

Here w(a) is the set of values of the parameters of the architecture a. DTtrain and DTval are the training and validation split of the dataset DT that characterizes the imaging task T. L is an appropriate loss function. Note that w*a and aT* are respectively the optimal parameters of a and optimal architecture aT* under L given DT.

Thus, our design problem is fully defined by three components: an architecture (a); a task (T) and its corresponding a dataset (DT); and a loss function (L).

Section 2: The optical architectures (a) and the search space of architectures (A)

Section 2.1: Definition of a physical optical system.

In this section we introduce the idea of a physical optical system (Fig.2). We first define a physical cartesian coordinate system x, y, z.

Fig.2 Conceptualization of an optical architecture.

We define z as the optical axis along which the light propagates. Each plane orthogonal to z is defined as a z-plane; The electric field across each z-plane is defined as Ez(x,y,λ,s), where λ corresponds to the wavelength of the light, and s corresponds to an independent light source. We define four special z-planes, z0, z1,z2, and z3. z0 is the input z-plane. We define an input electric field E0 at z0. E0 is fully described by the illumination light source. Between z1and z2 we model the volumetric object being imaged (for a thin object z1=z2), s.t. E1 is the input light field to the object and E2 is the output light field from the object. Last z3is the measurement z-plane, where a detector is placed. The light source, the object, and the detector are defined by the task T and do not contain any learnable parameters. All learnable parameters are related to the “optical elements” placed between z0and z1 and z2 and z3. We next introduce the concept of an optical element.

Section 2.2: Definition of an optical element

An optical element is defined as a transfer function Eout=fi(Ein|wi) where, Ein and Eout are the input and output light fields to the element. wi=(w0,w1,w2,…) are the parameters of the optical element. A typical optical element consists of light matter interaction process and a light propagation process through a medium. Thus, the input z-plane of an optical element is the same as the output z-plane of the previous optical element.

Section 2.3: Definition of the object volume

The object volume, h, is defined as a special optical element with two inputs, the input electric field and a 3D object xj, and no learnable parameters.

Eout=h(Ein,xj)

Section 2.4: Definition of an architecture, a

An optical architecture is defined as -and fully explained by- two sequences of optical elements.

a=f1,f2,f3,…,g1,g2,g3,…

The first sequence, f1,f2,f3,…, represents the illumination optics, and the second sequence, g1,g2,g3,…, represents the imaging optics.

Section 2.5: Definition of the search space A

The search space A consists of all possible combinations of sequences of optical elements.

Section 3: The imaging task (T) and its corresponding dataset (DT)

Section 3.1: Definition of a task

We define an imaging task as a function or a procedure T from a simulated 3D object xj∈X to its corresponding 2D target output light intensity field yj on an optical sensor, S, s.t.

yj=Txj | E0,S

As explained in section 2.1, E0 is the input electric field at the source z-plane z0. S represents the sensor; S is defined as a function that transforms the electric field E3 (at the measurement plane z3), to a digital image yj.

yj=SE3

X is the distribution of expected 3D objects.

Section 3.2: The dataset DT

We use T to simulate yi given xi (see section xx for examples) so that we can generate a dataset of paired inputs and outputs to solve the optimization problem in Section 1. Given PT to generate DT={x1,y1,x2,y2,…,xN,yN} we follow the steps below.

Step 1: Simulate a randomly drawn instance xj∈X such that each voxel contains all optical properties relevant to the task T.

Step 2: Use T to simulate the corresponding yj.

Step 3: Repeat steps1 and 2 for each data point.

Section 4: The loss function, (L)

We define the loss function over the dataset DT' as,

La,wa,DT'=1DT'0xj,yj∈DT'lossSGhF(E0),xj,yj

Here, F.=fNfN-1…f1.… and G.=gNgN-1…g1.… are respectively the transfer functions of the illumination and imaging optics. They contain the learnable parameters. loss(.) is a distance function (like MSE or L1) between the response of the learnable optical system and the target. DT'0 is the cardinality of DT'.

Section 5: Example tasks

In this section we define a set of tasks to demonstrate our OAS approach.

3.1. Coherent imaging of a thin amplitude object: We first consider the simplest imaging task that relays one image plane to another with a desired magnification m. Traditionally this is performed using two lenses with different focal lengths placed at what is known as a 4f configuration.

We first define a thin object xi as a random amplitude mask, s.t. each spatial location xi(x,y)∈R, and x,y ∈[-r,r]. Note that the mask is defined over a 2r×2r region centered at the optical axis. The corresponding intensity field at the output, yi is then given by,

yix,y=Txjxm,ym | E0=U,S=SIdealCam

Here the electric field from the source is a uniform field, U, and the sensor is an ideal camera,

SIdealCamE3~PoissonE32.